ITK-Lung: A Software Framework for Lung Image Processing and Analysis

2 Specific Aims

One of the most significant hurdles to leveraging imaging innovations in adopting more quantitative clinical practices and exploring additional novel research pathways is the availability of accurate, robust, and easy-to-use image analysis tools.  Historically, the research and clinical communities (and their overlap) have significantly benefited from computational image analysis packages, particularly those softwares which have been tailored for specific application domains. Although several such established packages exist for neu- roimaging research (e.g., FSL, FreeSurfer, AFNI, SPM), no such general package exists for pulmonary imaging analysis. The primary goal of this project is to develop a robust, open-source image analysis toolkit and dissemination platform specifically targeted at the pulmonary research community. Given the significant efforts to make lung imaging datasets publicly available (such as LIDC and RIDER), this contribution would be innovative as it would meet a critical need through a first-of-its-kind software package for multi-modality lung image analysis.

Although methodological research is continually being presented at conferences and published in various venues, the unfor- tunate reality is that much of this work exists strictly in “advertisement” form. Oftentimes the underlying code is unavailable to other researchers or is implemented in a limited manner (i.e., strictly as proof-of-concept software). Frequently, crucial pa- rameter choices are omitted in the corresponding publication(s) which makes external implementations difficult. In addition, the data used to showcase the proposed methodologies is often limited to carefully selected snapshots for publication purposes which might not be representative of algorithmic performance.  Finally, many of these analysis methods are patented and/or integrated into proprietary commercial software packages which limits accessibility to researchers.

As a corrective alternative, this project brings together leading expertise in lung imaging research at the University of Penn- Sylvania (Penn) and University of Virginia (UVa) to develop, evaluate and deploy under community support an open-source software toolkit targeted for pulmonary imaging research. As principal developers of the popular, open-source ANTs, ITK-SNAP and ITK pack- ages, we have extensive experience in the development of well-written software and propose to make a similar impact in the pulmonary community with this project. Specifically, we plan to provide core algorithms for specific pulmonary image analysis tasks across multiple modalities, many of which we have included with previous publications.  These basic tasks include intra- and intermodal pulmonary image registration, template building for cross-sectional and longitudinal (i.e., respiratory cycle) analyses, functional and structural lung image segmentation, PET imaging, perfusion analysis, and computation of quantita- tive image indices as potential imaging biomarkers. These efforts would facilitate other NIH-sponsored projects which interface specific pulmonary algorithms (e.g., CT nodule detection) with clinical and research applications.  In addition to the software, we will provide scripts, documentation, and tutorial materials consistent with open-science principles.  Formally, this project is defined by the following specific aims:

•  Specific Aim 1:  Develop ITK-Lung, a set of open-source software tools for CT, PET, proton, and He-3 pulmonary computational  analysis.  These open-source software tools based on selected algorithms will specifi- cally target pulmonary image analysis and comprise core application functions such as inspiratory/expiratory  registra- tion, ventilation-based segmentation, lung and lobe estimation, airway and vessel segmentation, PET imaging, perfusion analysis, and calculation of clinical indices for characterization of lung development and pathology. As a complement to these software development efforts, CT and 1H MRI multi-atlas libraries will be provided as open data, complete with the corresponding lung, airway, vessel, and lobe segmentations according to modality to facilitate the employment of atlas-based algorithms on other data sets.  In addition, we will generate optimal intensity/shape templates from each library for use as common coordinate frameworks for more localized (i.e., voxelwise) analyses.  Both sets of data will be provided with the scripts used to produce them in order to permit user-reproduction  of the results.  As developers of several leading open-source applications for image segmentation and registration, we know firsthand that the impact of a particular technological innovation greatly depends on the availability of an easily accessible software implementation. The proposed software framework will tie together all of the capabilities of the project’s developed methodology in the form of programmable workflows and provide a seamless user experience through a full featured graphical user interface. Interactive functionality will extend beyond the ability to steer segmentation and registration pipelines to include tools for evaluation and visualization of processed results.

•  Specific Aim 2: Validate and disseminate the developed ITK-Lung resources by leveraging use cases from a broad network of partner investigators representing the state-of-the-science in lung imaging research. This aim will evaluate and refine the proposed framework within the real-world context of pulmonary research being carried out at Penn and UVa in addition to various partner sites serving as secondary beta testers. We will disseminate the results of the project through open-source distribution of the software, atlases and documentation, online user support, and conduct of hands-on training workshops.

3 Research Strategy

3(a) Significance

3(a.1) The importance of image analysis tools for research and clinical investigation. The increased utilization of imaging for both research and clinical purposes has furthered the demand for quantitative image analysis techniques. The use of these computational techniques is motivated by the need for less subjectivity and more standardization in medical image interpretation, increased speed and automation in diagnosis, and greater robustness and accuracy for determining biological correlates with imaging findings.  For example, in the area of pharmaceutical development and testing, imaging biomarkers are crucial.  In order to determine fundamental study parameters such as drug safety and effectiveness, quantitative assess- ments derived from imaging measures must be objective and reproducible [1] which is often difficult without computational aid given the intra- and inter-reader variability in radiological practice [2, 3]. Additionally, the exciting possibilities associated with “big data” and the potential for improvement in individualized, evidence-based medicine has also increased the need for sophisticated data transformation and machine learning techniques.

3(a.2) Open-source as an essential attribute of high-impact image analysis toolkits. Well-vetted and publicly avail- able software have transformed targeted research fields. For example, the neuroscience community has greatly benefited from highly evolved software packages such as FreeSurfer [4], the FMRIB Software Library (FSL) [5], the Analysis of Functional NeuroImages (AFNI) package [6], the Statistical Parametric Mapping (SPM) package [7], and several others.

However, despite the important implications for the pulmonary imaging community, no such analogous set of tools exist for multi- modal pulmonary-specific research. Such an original software package would potentially have an immediate and significant impact.  Indeed, in a recent review of CT- and MRI-derived biomarkers for pulmonary clinical investigation, the authorial consensus is that “[the absence of] universally available image analysis software” is a major hinderance to more widespread usage of such imaging biomarkers [8].

Medical image analysis libraries (e.g., the NIH-sponsored Insight ToolKit) provide extensive algorithmic capabilities for a range of generic image processing tasks. However, tailored software packages for certain application domains (e.g., lung image anal- ysis) do not exist despite the vast number of algorithms that have been proposed in the literature (most of which are not avail- able to the public).1   Importantly, the goals of this project would significantly support the National Library of Medicine’s (NLM) open-source directives in that all software proposed in the project would be developed using the established Insight ToolKit’s coding and testing standards with the specific objective that all project code would be contributed for inclusion in future versions of the Insight ToolKit (ITK) as we have done in the past. NLM’s position on open-source stems from the documented benefits within the targeted communities for which the software is developed and supported. In addition to increase in research output, open-source permits trainees and researchers to learn specific computational techniques in a social environment [9]. This, in turn, provides motivation for user-based support including potential contribu- tions such as bug fixes and feature additions.  Additional analyses have shown the tremendous cost savings that open-source software yields [10].  Furthermore, open-source development and distribution within a large, and well-invested community (such as ITK) takes advantage of Linus’s law, i.e., “given enough eyeballs, all bugs are shallow,” for producing robust software.

3(b) Innovation

3(b.1) Open-source pulmonary imaging algorithmic innovation.  Given the lack of open-source solutions for multi- modality pulmonary image analysis, this project would produce the first-of-its-kind processing and analytic platform for per- forming such research.  Similar to the brain-specific algorithms provided in our ANTs (Advanced Normalization Tools) toolkit [11], our project would include several essential algorithms for analyzing lung images from different modalities, including CT, PET, and 3He and 1H MRI. A large number of algorithms have been proposed in various technical venues, but these are mostlu in the form of textual descriptions without accompanying software. In contrast, we will provide well-vetted and easy-to-use implementations of specific robust methodologies for pul- monary image analysis, many of which have been developed by our group. To facilitate the usage of these algorithms, we will provide documentation, including self-contained online examples, tutorials, and hands-on training workshops. We rec- ognize that the methodological depth of the field is extensive and implementing even a small portion of the total number of algorithms would be prohibitive. However, the carefully curated selection of algorithmic implementations provided through this project have been extensively discussed in the literature and have delivered consistently excellent performance in our clinical collaborations.  The availability of such implementations will offer a unique clinical utility to the community as well as a performance benchmark and baseline platform for future algorithm developers.

3(b.2) Use case studies with leading pulmonary research scientists.   Another innovative component of the project is the inclusion of an extensive set of use cases from leading pulmonary research groups, with regression testing performed using different image acquisition protocols, equipment, etc. to ensure quality and robustness of the software and processed data.  The use of separate, independent testing sites will increase the value of the tools produced by ensuring that their success is not specific to the particular source of data.  This will increase the generality of the developed resources and thus ease dissemination to the wider community. Toward this end, the

1 Several competitions have been held in recent years focused on the processing and analysis of lung image data (e.g., VOLCANO09—nodule detection, EMPIRE10—registration and motion estimation, LOLA11—lung and lobe segmentation, and VESSEL12—vasculature segmentation). To the best of our knowledge, the vast majority of the proposed algorithms are not publicly available. Other pulmonary imaging efforts, such as LIDC and RIDER, have amassed large amounts of imaging data but available software support is limited to organizational tasks specific to those databases.

Functionality 	Papers

spatial normalization		  [13–20] template generation			[21] lung segmentation	[19, 22–25] lobe segmentation		[24, 25]

airway segmentation	[26]

functional segmentation	[21, 27, 28]

feature indices	[22, 29, 30]

Table 1: Research contributions from our group demonstrating pulmonary multi-modal image domain expertise.

real-world use cases were solicited representing as broadly as possible the requirements of the community as well as the multiple modality and algorithmic variations which commonly occur.

Tutorial materials, data, and example scripts drawn from these studies will be provided to the public for any interested researcher to apply to their own data. Any clinical findings of interest will be published in traditional venues (e.g., Chest).  In addition, we will provide all the quantitative analysis scripts as a companion release for the paper (e.g., see previous similar offerings from our group [12, 13]). A clinically-based evaluation of these tools will provide insight into the specifics of certain pulmonary pathologies and also offer a reproducible mechanism for using the tools created in this project.

3(c) Research design

3(c.1) Preliminary data

Major progress toward the proposed platform, demonstrating project feasibility, has already been reported by our group (cf Table 1).  These publications not only document methodological novelty of the work but also describe their subsequent usage for clinical research studies of small to large cohorts.  Much of this innovation has been encoded in prototypical form in the ANTs processing toolkit, as described below, to allow for continued use, potential future improvements, and reproducibility.

3(c.1.1) Generic ANTs core tools for image analysis and processing.   The Advanced Normalization  Tools (ANTs) package is a state-of-the-art, open-source software toolkit for image registration, segmentation, and other basic medical image analysis functionality [11].  Several core programs comprising portions of the proposed pulmonary imaging analysis software framework have been created and made available within ANTs (and either simultaneously or subsequently made available in ITK). However, these programs have more general application and require pulmonary-specific  tuning for the tasks targeted by this project.  The following list comprises available functionality proposed for tuning, subsequent exten- sions, documentation, tutorial generation, and the creation of easy-to-use bash scripts for large-scale processing of pulmonary imaging data:

ANTs image registration.  One of the most important innovations in medical image analysis is the development of image registration techniques capable of mapping the highly complex variations seen in human anatomy. Our team is well-recognized for seminal contributions to the field that date back to the original elastic matching method of Bajcsy and co-investigators [14–

16]. Our most recent work, embodied in the ANTs open-source, cross-platform toolkit for multiple modality image processing, continues to set the standard in the field for lung [17], brain [18], and cardiac imaging [19]. ANTs not only encodes the most advanced results in registration research, notably the Symmetric Normalization (SyN) algorithm for diffeomorphisms  [20], but also packages these within a full featured platform that includes an extensive library of similarity measures, transforma- tion types, and regularizers. Recently, a thorough comparison with the original SyN algorithm was performed using a B-spline variant [12].  This evaluation utilized multiple publicly available, annotated data sets and demonstrated statistically signifi- cant improvement in label overlap measures. As part of that study, we produced the scripts aMib_2;Bbi`aiBQMavLXb? and aMib_2;Bbi`aiBQMavLZmB+FXb? which provide a simple interface to our normalization tools for brain-specific normaliza- tion and are two of the most widely used scripts in the ANTs toolkit. Similar to the developments that we are proposing, these scripts were extensively modified to serve as a follow-up entry into the EMPIRE10 lung image registration challenge where B-spline SyN performed better than its original counterpart on pulmonary data [21], which had been the official top ranked entry since the inception of the challenge.

Multi-modality template generation. Given the variability in anatomical shape across populations, generating population- or subject-specific optimal shape/intensity templates significantly enhances study potential [22, 23].  First, an average template is estimated via a voxelwise mean of all the individual subject images. This estimate is iteratively updated by registering each image to the current template, performing a voxelwise average

to create a new estimate, and then “reshaping” this template based on the average inverse transformation which “moves” the template estimate closer to the group mean—see Figure 1 for a cohort-specific multi-modality brain template for females in the age range 50–60.  This functionality has proven to be a highly valued component of the ANTs toolkit, with significant community adoption, for performing neuroimaging  research (e.g., [13, 24–28]).   The same technology specialized to lung imaging will accelerate the translation to the pulmonary domain of voxelwise studies that have transformed the neuroimaging field, and may prove equally valuable and impactful to the pulmonary research community [29].

Bayesian segmentation with spatial and Markov Random Field priors. Early statistically-based segmentation work appropriated  NASA satellite image processing  software for classification  of head tissues in 2-D MR images [30].   Follow- ing this work, many researchers adopted statistical methods for n-tissue anatomical brain segmentation.  The Expectation- Maximization (EM) framework is natural [31] given the “missing data” aspect of this problem.  Core components of this type of work include the explicit modeling of the tissue intensity values as statistical distributions [32, 33] and the use of Markov Random Field (MRF) modeling [34] for regularizing the classification results [35]. Spatial prior probability maps of anatomical structures of interest are also employed within this framework [36, 37]. Although this particular segmentation framework has significant application in the neuroimaging domain, it is also relevant to other domains including functional ventilation of the lung [38]. However, despite the numerous developments which have been proposed over the years within this area, there are only a few actual software implementations. This deficit inspired us to create our own Bayesian segmentation framework [39] (denoted as Atropos), which we have made publicly available within ANTs and has proven highly effective in quantification of functional lung imaging data [38, 40–42].

N4 bias correction. Critical to quantitative processing of MRI is the minimization of field inhomogeneity effects which pro- duce artificial low frequency intensity variation across the image. Large-scale studies, such as ADNI, employ perhaps the most widely used bias correction algorithm, N3 [43], as part of their standard processing protocol [44]. In [45], we introduced “N4,” which incorporates both an enhanced fitting routine (including multi-resolution capabilities) and a modified optimization formulation that produces a significant improvement over N3 in performance and convergence behavior on a variety of data.  N4 has since become the new standard in the field.

Joint label fusion for multi-atlas segmentation. Joint label fusion (JLF) is the current state-of-the-art for propagating expert labelings from a reference atlas library onto new instances of unlabeled data.  Image registration is used to align the atlas library (images plus segmentations) to a common space. A statistical model is then used to combine the “guesses” from all the normalized atlas labels to provide a “best guess” estimate of the target labeling.  Several such algorithms have been developed and much effort has been devoted to determining relative performance levels—see, for example, the recent MICCAI

2012 Grand Challenge and Workshop on Multi-Atlas Labeling. The joint label fusion (JLF) algorithm of [46, 47] from our group is widely recognized as among the best performing, having placed first in the MICCAI Grand Challenge.  JLF is capable of predicting anatomical labels with accuracy that rivals expert anatomists [48], and has proven its effectiveness in multiple domains [49], [13], [49]. Importantly, we have successfully extended JLF to the challenging problem of applying prior-based information to lung and lobe segmentation [50].

Spatially adaptive denoising. Denoising is critical for data “cleaning” prior to subsequent processing such as segmentation or spatial normalization.  ANTs implements a state-of-the-art spatially adaptive version to patch-based denoising recently proposed in [51].

Field-leading open-source implementations.  The previously described core software functionality, as well as several others, have been part of ANTs and ITK development efforts for more than a decade. The deficiency of publicly available tools within the neuro- science community was the original motivation for the inception and continued development of ANTs. As a result, our team is well-recognized for our many open-source advancements, including important contributions to the field of image registration outlined earlier. Indeed, ANTs-based image registration serves as the basis for the registration component of the latest version of the National Library of Medicine Insight Toolkit programming library (http://www.itk.org), the leading open-source platform for medical image analysis. The combination of state-of-the-art algorithms and feature-rich flexibility has translated to top-placed rankings in major independent evaluations for core elements of the ANTs toolkit:

•  SyN was a top performer in a fairly recent large-scale brain normalization evaluation [18].

•  SyN also competed in the Evaluation of Methods for Pulmonary Image REgistration 2010 (EMPIRE10) challenge [17], where it was the top performer for the benchmarks used to assess lung registration accuracy and biological plausibility of the inferred transform (i.e., boundary alignment, fissure alignment, landmark correspondence, and displacement field topology). The competition has continued to the present and SyN has remained the top-ranked algorithm.

•  The joint label fusion algorithm of [46, 52] (coupled with SyN) was top-ranked in the MICCAI 2012 challenge for labeled brain data [53] and in 2013 for labeled canine hind leg data [54].

•  The multivariate template capabilities in ANTs were combined with random forests to win the Brain Tumor segmentation

(BRATS) competition at MICCAI 2013 [23].

•  A B-spline variant of the SyN algorithm [12] won the best paper award at the STACOM 2014 workshop for cardiac motion estimation [19].

3(c.1.2) Neuroimaging with ANTs as a model for the pulmonary community. ANTs takes advantage of the mature Insight Toolkit in providing a powerful framework for building scripts and programs to facilitate processing of large neuroimaging studies.  In particular, ANTs has developed a very large user base by making openly available a complete suite of fundamental processing capabilities, covering brain normalization [55, 56], brain template generation [22], skull-stripping or brain extraction [13, 57], prior-based brain tissue segmentation [55], cortical thickness estimation [13, 58], brain tumor segmentation [23], and cortical labeling [46, 52].  These tools have been wrapped in easy-to-use, well-documented shell scripts that are accompanied by online self-contained examples with developer-tuned parameters and are compatible with the major cluster systems (e.g., SLURM, SGE, and PBS).  This project will implement a similar strategy to support the lung imaging community, as demonstrated by the following software utilities that have already begun to find widespread use among pulmonary research groups:

Figure 1: Core processing tools that have made the ANTs package one of the most popular neuroimaging toolkits. Fundamental processing tasks such as image registration, template generation, bias correction, denoising, intensity-based segmentation, and joint label fusion are first-in-class software components which have been utilized for neuroimaging tasks such as brain extraction and cortical thickness estimation. The target applications of these core tools have an immediate analog for lung-specific tasks such as lung and lobe segmentation.

•  intra-modal lung registration [12] (https://github.com/ntustison/antsCtLungRegistrationExample),

•  inter-modal lung registration [12] (https://github.com/ntustison/ProtonCtLungMaskRegistration),

•  functional lung segmentaiton [38] (https://github.com/ntustison/He3LongitudinalAnalysis), and

•  lung and lobe segmentation [50] (https://github.com/ntustison/LungAndLobeEstimationExample).

3(c.2) Specific Aim 1: To develop ITK-Lung, a set of open-source software tools for CT, PET, 1H, and 3He pulmonary computational analysis

The envisioned open-science tool set for pulmonary image analysis consists of software, processed data to illustrate the use of the software, and the ability to evaluate and visualize user-generated results.  With this comprehensive offering, the goal of this project is to help the pulmonary imaging research community on a much deeper level than simply providing a set of programs.  In order to facilitate engagement on the part of the community, we are proposing a multi-faceted approach with ITK-Lung.  The main component will be the core software tool set described in Sub-Aim 1a which would permit large-scale processing of multi-modal pulmonary image data.  To illustrate the use of the software, allow for processing of other public and private data sets, and provide baseline data for algorithmic comparison, the second component will involve the release of CT and 1H MRI annotated atlas libraries, corresponding templates, and data-generating scripts as described below. The third component will be significant extensions to the well-known ITK-SNAP software for an enhanced user experience through a full featured graphical user interface to support

Figure 2: ANTs based core processing tools for multi-modal lung imaging studies. Using ANTs processing tools, our team has developed several lung-specific extensions such as ventilation-based segmentation, lung and lobe estimation, and multi-modality pulmonary template build- ing. Although each of these extensions requires significant additional development and tuning, a robust and generic software foundation ensures that these extensions are of high quality and are readily adapted to the pulmonary image domain.

interactive tuning of parameters and interrogation of processed results.

3(c.2.1) Sub-Aim 1a will expand the ITK/ANTs open-source libraries by implementing currently unavailable lung-specific algorithms. Many important algorithmic categories implementing fundamental lung image analysis tasks do not currently exist in any comprehensive, publicly available package. This is despite the fact that new algorithms for lung image analysis are frequently reported in the literature.  An extensive survey concentrating on the years 1999–2004 is given in [59] which covers computer-aided diagnosis of lung disease and lung cancer in CT (i.e., detection and tracking of pulmonary nodules) and provides an overview of the many relevant segmentation methods for pulmonary structures.  Although many algorithms existed at the time, continued technical development has only increased the number of available algorithms. However, despite the continued reporting of pulmonary image analysis algorithms, there is no corresponding increase in algorithmic availability. Additionally, a key problem in the pulmonary image analysis community is that the lack of publicly available tools translates directly into a paucity of baseline performance standards with which researchers can compare their own algorithms [60].  This project constitutes a specific and overdue response to this major deficiency in the field.

Toward this end, a select set of tools with a track record of good performance, spanning the range of core functionality and designed to facilitate expansion, will serve the pulmonary research community as a well-vetted quantitative resource and baseline for future algorithmic development.  Table 2 comprises the proposed functionality for multi-modal lung image analysis that would be incorporated into ITK-Lung in addition to further enhancements to the registration and segmentation capabilities described in preliminary work.  Using ANTs core tools with lung-specific modifications, we have produced prototypical implementations, complete with preliminary documentation and github examples, for several of the proposed processing capabilities, as described below.

Atlas-based lung segmentation. Identification of anatomical structure in lung images is often a crucial preprocessing step for quantification of morphological features or ventilation information from functional images. Quantitative regional analysis typically requires the delineation of lung and lobar anatomy. Although much algorithmic research for lung segmentation has been reported in the CT literature [61], co-opting such technologies is complicated in MRI by issues such as RF coil inhomo- geneity, presence and resolution of structural detail, and the absence of a physically-based intensity scaling.

We recently proposed a multi-atlas approach for automatically segmenting the left and right lungs in 1H MRI [50]. Multi-atlas approaches to segmentation have proven highly successful in multiple domains [46, 52] and these methods translate readily to pulmonary applications.  Whereas many current strategies for lung image segmentation employ low-level processing techniques

Functionality 	CT 	1H MRI 	3He MRI

spatial normalization template generation lung segmentation lobe segmentation airway segmentation vessel segmentation

functional segmentation feature indices

Table 2: Core functionality proposed for development and evaluation in the project categorized by modality. One of the motivations for the collaborative use cases as a specific aim is the inevitability that other lung-specific algorithmic needs will be identified and will be added to the functionality developed and offered as part of this project. It should also be noted that some modality-specific modifications will be required. For example, our lobe estimation approach works well for 1H MRI where no internal anatomical features are available for refinement. This lobe estimation strategy can be directly applied to CT in providing spatial prior maps for subsequent subject-specific refinement.

based on encodable heuristics, consensus-based strategies, in contrast, optimize the prior knowledge applied to a specific seg- mentation problem (cf Figure 2). The evaluation of our proposed method [50] demonstrated excellent performance with Jac- card overlap for the left and right lungs measuring 0.966 0.018 and 0.970 0.016, respectively. Further work for this project includes extension to CT datasets with a particular emphasis on segmentation in the presence of lung pathology that will incorporate the data from the proposed multi-atlas CT library. [FIX #1: explain in a few sentences how lung pathology will be addressed] [FIX #2: add text describing EXTENSION to PET]

Atlas-based lobe estimation.  For regional investigation of certain lung pathologies and conditions, it is often useful to quantify measurements of interest within more localized regions, such as the lobes.  However there is little (if any) usable in- formation in 1H MRI for image-based lobar segmentation which has led to alternative geometric subdivisions which are ad hoc, non-anatomical, and do not adequately address intra- and inter-subject correspondences.  Nevertheless, we can take advan- tage of inter-subject similarities in lobar geometry to provide a prior-based estimation of lobar divisions using a consensus labeling approach (cf Figure 2).  Specifically, to generate the lobe segmentation in a target 1H or CT lung image, we register the same-modality atlas set to the target image (given the general increased robustness of intra-modality vs. inter-modality image registration) using the B-spline SyN registration approach described earlier [12]. Subsequently, we warp the set of lobe label images to the target image using the atlas-to-target transformation.  This process will be illustrated publicly as part of the project using the open-data multi-atlas CT and 1H atlas libraries created as part of Sub-Aim 1b. Since we have no intensity information inside the target lung mask and CT atlas lung masks, a simply majority voting strategy is used to generate the consensus labeling for the target image. Following this step, we remove any labelings outside the lung mask and assign any unlabeled voxels with the label closest in distance to that voxel. Additional details can be found in [50] where we showed that lobar overlap measures in 1H MRI were on par with state-of-the-art methods for CT images, in which fissure information is actually visible (left upper: 0.882 0.059, left lower: 0.868 0.06, right upper: 0.852 0.067, right middle: 0.657 0.130, right lower: 0.873 0.063). We will extend this framework to pulmonary CT in providing spatial prior probability maps de- rived from image-specific CT data features such as fissures, airways, and blood vessels for data-driven, subject-specific lobe segmentation [62]. [FIX: add text describing EXTENSION to PET]

Ventilation quantification.   The ability to classify areas of varying degrees of venti- lation represents a basic need in pulmonary functional analysis.  In [38], we presented an automated algorithmic pipeline for ventilation-based partitioning of the lungs in hyperpolarized 3He and 129Xe MRI. Given a whole lung mask (see  Atlas- based lung segmentation ), the original pipeline performs MR inhomogeneity correction followed by Bayesian segmentation with an MRF prior.  Without ground truth data for evaluation, we used a consensus labeling approach [63] to simultaneously estimate the true segmentation from given “raters” which included the segmentation from our automated approach and the manual tracings of three trained individuals. In terms of combined specificity and sen- sitivity, our algorithm demonstrated superior performance with the added benefit of being reproducible and less time-consuming.  Future enhancements to this pipeline will include the incorporation of 1) an iterative bias-correction/segmentation scheme that should yield improved optimization solutions and 2) a better performing denoising protocol based on the patch-based method described in [51].

Multi-modality  lung template  construction.   Because  the template  construction  algorithm  described  in [22] was originally developed for proton MRI and neuroimaging applications, it will be modified in this project to additionally admit CT and PET data and further adapted for specialized use in studies of the lung.  The latter development will build on our recent innovations in diffeomorphic  registration technology that has led to a Symmetric Normalization B-spline variant, which will be extended and refined to include patch-based  similarity metrics suitable for minimizing  the

computational footprint of problems involving very large images of the kind commonly encountered in lung studies while at the same time providing accurate normalizations [12] for pulmonary data [21]. [FIX: additional text addressing considerations for MRI and PET data and/or multi-modality data???]

Feature indices.  Imaging biomarkers for characterizing emphysema in CT have have been well researched, although there are ample opportunities to refine these methods as well as to introduce more advanced approaches.  Examples of the latter include texture analysis for identifying the centrilobular and groundglass opacities and fractal and connectivity approaches to differentiate centrilobular from panlobular emphysema.  The available indices for CT image analysis can roughly be divided into those that characterize the pulmonary parenchyma:  volumetric tissue (e.g., [64, 65]), distribution of low attenuation ar- eas (LAA) (e.g., [66, 67]), cooccurrence and run-length matrix features (e.g., [68, 69]), attenuation statistics (e.g., [70, 71]), deformation measures (e.g., [72, 73]), and stochastic fractal dimension features (e.g., [68, 71]); and those that characterize the airways (e.g., [74–76]). The former are important for subjects with an emphysematous component of disease, whereas the lat- ter are important for subjects with a bronchitic disases component. An important benefit of this project’s multi-modality scope is that many of these measurements can also be directly applied to discriminative analysis using 3He MRI and PET for a variety of lung diseases. These indices can also be studied not only at any particular single time point, but also for changes with time. The addition of quanti- tative morphologic measurements of the airways provides an assessment of the contribution of airway changes to chronic lung disease.

Airway segmentation.  Lung airway morphology has been previously utilized as a biomarker for disease characterization, but there are many other potential uses motivating the inclusion of airway segmentation in any pulmonary image analysis toolkit.  In an evaluation of 15 airway segmentation algorithms [77] for the Extraction of Airways from CT challenge held in 2009, the top 2 performers were the algorithms of [78] and [79] with the latter being one of the more conservative algorithms and the former being more prone to false positives. Our plan is to provide an implementation  of [78] which employs a sharpening of the airway branching edges and adaptive cuboid volumes to realize a region-growing airway segmentation approach. We will then augment this functionality with complementary aspects of our previous work [80] for removing leakage path candidates.

Vessel segmentation. In contrast to much of what is described in the literature, there exist key contributions for pulmonary vessel segmentation available to the public, specifically the algorithms described in [81].  Similar to much of the research for enhancement of vessel-like morphology in different anatomies, the authors employed Hessian-based filters for deriving a “ves- selness” function. Their approach ranked consistently among the top performers in the VESSEL12 challenge organized in con- junction with the IEEE International Symposium on Biomedical Imaging (ISBI 2012). Given that the first author is a regular ITK contributor, the corresponding code will be easily integrated into our platform.

Perfusion analysis. A nonparametric deconvolution technique for quantifying cerebral blood flow was first presented in [82] and subsequently extended for use in assessing pulmonary blood flow [83, 84].  This deconvolution strategy was recently implemented by our group for a separate lung imaging study but will be modified for this project to automatically select the arterial input region and volume of interest.

PET imaging. [FIX: I suggest we focus on intermodal PET/CT and PET/MR co-registration functionality.  Should this be separate subsection?  Can you rework “Multi-modality lung template construction” subsection, possibly renaming it “Multi-modality lung image registration,” and integrating intermodal co-registration discussion into existing template/registration text (and explicitly highlighting features/advantages of multi-modal algorithm vs. separately building uni-modal templates)?]

Graphics processing unit acceleration. In preliminary work, we have successfully used OpenGL shader language to per- form image processing operations, such as computation of similarity metrics between images or extraction of features from images, on the graphics processing unit (GPU) at real-time speed.  GPU requirements for ITK-Lung will be very modest and would be met by virtually any mid-range desktop computer sold today (e.g., a mid-range iMac).  We plan to integrate both affine and deformable registration with GPU acceleration into ANTs. The three components of diffeomorphic deformable reg- istration in ANTs that account for over 95% of computation time are: 1) computation of the gradient of the similarity metric (e.g., mutual information, cross-correlation);  2) smoothing of the gradient using Gaussian kernels; and 3) image and vector field interpolation. All three components are highly parallelizable, and we will write OpenGL shader code for performing these operations on the GPU, analogous to the code currently used for metric computation in GPU-based affine registration in ANTs.

Computational power.  With the proposed GPU acceleration of image registration, we expect that even the most complex registration-dependent  problems will be possible to solve on a desktop computer in under 10 minutes.

This estimate is based on  preliminary work in which we have developed a CPU-based accelerated version of the ANTs

deformable registration algorithm that is approximately 15 times faster in typical problems than the version currently in use. This means that on a typical 8-core CPU, the registration of standard 512 512 ∼500 voxel lung images can be completed in under

20 minutes instead of the 1-2 hours it would take on a single CPU core using the current production version of ANTs.  This dramatic improvement was made primarily through 1) use of more efficient metric gradient computation al- gorithms, 2) use of single instruction multiple data (SIMD) parallelization on Intel CPUs, and 3) extensive code profiling and optimization. With the move to the GPU, we expect to speed up registration by another factor of 4-5 times. Therefore, the vast majority of ITK-Lung users will not need to have access to high-performance computing clusters or other expensive hardware.

However, for power users with very large datasets or datasets that require more compute-demanding  parameters or higher- resolution registration, we will also provide an alternative offline processing mode, in which the registration tasks are executed

on an external high-performance computing resource.  In this mode, the ITK-Lung GUI (see Sub-Aim 1c) will provide a user interface front-end to available processing pipelines running remotely. It will send data to the external resource over the network, monitor and control pipeline execution, and receive results when processing is complete. This remote pipeline execution mode will additionally support other highly computationally intensive tasks as well as other data processing tasks that may be added in the future. The external resource may be either a physical Linux cluster or a virtual one hosted on the Amazon Web Services (AWS) cloud.  For AWS, we will use the MIT StarCluster software (http://star.mit.edu/cluster) to launch a virtual cluster directly from the ITK-Lung GUI on demand. We will take advantage of the GPU enabled AWS instances, which would allow the speed-ups described above to be combined with parallel execution across multiple hosts. The advantage of using AWS is that a power user would not need to purchase and configure expensive hardware—they just register for an AWS account.  While this would incur costs for computational time used, these costs would be a fraction of the cost of the required hardware.  For instance, even with the most demanding datasets and parameters, it is hard to envision a processing task that would require more than an hour when distributed across 40 GPUs on AWS, the cost of which is currently $26. Data transfer to AWS will incur negligible costs ($0.01 per GB), particularly since data may be uploaded at reduced resolutions sufficient for many processing tasks.

3(c.2.2) Sub-Aim 1b will provide two annotated multi-atlas libraries, one for CT and another for 1H MRI. The corresponding group templates will also be provided along with the scripts to produce the results using ITK-Lung. As a comple- ment to the open-source software provided in Sub-Aim 1a, we will generate atlas libraries for both CT and 1H MRI acquisitions. As indicated above, the availability of such atlases as anatomical priors are prerequisite to application of the current state-of-the-art in robust and accurate lung and lobe segmentation within their respective modalities. Additionally, the accompanying annotations provide an open-data platform for quantitatively assessing other lung and lobe segmentation algorithms.  Both libraries will consist of n = 30 different sub- jects to represent a range of lung size and shape. The CT atlas library will include lung, lobe, airway, and vessel segmentations, whereas the 1H MRI atlas library will include left/right lung segmentations and lobar estimations [50].  Along with the annotated data, we will provide the scripts and documentation to allow independent reproduction of the ITK-Lung results.  Finally, optimal group templates [22] for the two atlas libraries will be generated and made openly available to be utilized as common coordinate frameworks to support more localized (i.e., voxelwise) analyses.

3(c.2.3) Sub-Aim 1c will develop a graphical user interface (GUI) for running ITK-Lung on user data and eval- uating processed results.  ANTs will be the workhorse toolkit for the registration development effort in Aim 1, and the creation of a user-friendly GUI enabling interactive access for the first time to ANTs functionality will be a critical innovation toward a comprehensive registration solution for imaging studies of the lung.  The existence of the GUI will not only open ANTs to users without programming experience—which is expected to greatly expand its already considerable user base and in turn further increase its impact on the field—but will also significantly enhance the power of ANTs by allowing human/expert input to interactively tune parameters and intelligently initialize as well as steer the registration toward the solution that best satisfies both user and algorithmic constraints.  Equally important, the GUI will permit qualitative and quantitative assess- ment of the results produced by ITK-Lung as well as potentially other software, another much needed capability not currently available to the general community. Taking these considerations into account, we propose to base the GUI front-end to ITK-Lung on the ITK-SNAP multi-platform,  open-source tool for interactive user-guided medical image segmentation and data visualization [85], the development of which is led by project investigator Paul Yushkevich.  ITK-SNAP provides an effective combination of semi-automatic segmentation functionality based on active contours [86] and manual delineation functionality, put together into a compact and easy-to-learn GUI, that perfectly complements the automated segmentation functionality proposed for de- velopment in this project.  ITK-SNAP design emphasizes interaction and ease of use, with the bulk of the development effort dedicated to the user interface.  ITK- SNAP has thousands of users (there have been over 2000 downloads per month in the last year), and our 2006 paper on ITK-SNAP [85] has been cited over 1400 times (Google Scholar) in the context of various biomedical domains. ITK-SNAP will also be used in this project for manual labeling of the proposed lung atlases; it is already used for this purpose by many investigators.  Most crucially, we believe that our track record with ITK-SNAP as well as ANTs demonstrates our team’s commitment to producing high-quality research software and making it accessible to the wider re- search community through open-source practices, intuitive user interfaces, and outreach efforts. These strengths of the team will be applied to the software and data developed in the course of this project.

Several features will be added to ITK-SNAP to enhance visualization and quantitation for the registration and segmentation results of the lung-specific algorithms developed in the project.  Users will be able to edit and annotate these segmentations, modify registration transforms, and extract quantities both globally and regionally. Transforms will be modifiable via manual annotations (clicking corresponding landmarks, tracing curves, and/or painting regions) or by directly modifying transform parameters.  Quantitative parameters, such as volumes and strain tensors, will be available through the ITK-SNAP interface. Finally, existing ITK-SNAP functionality for segmentation visualization will be expanded to support evaluation of registration quality, including a dashboard of performance metrics, linked cursors identifying corresponding positions in multi-window configurations,  and fused data displays with adjustable blending.  These proposed enhancements to the software will be ex- tremely useful to the general imaging research community and not just those investigators targeted in this project. Thus, the impact of this work will be both immediate and broad on pulmonary-driven science and research.

3(c.2.4) Software engineering.   Both ANTs and ITK-SNAP development,  based on a solid foundation provided by the Insight Toolkit, utilizes open-source software engineering best practices, such as the use of Git version management software for collaborative development and easy branching and merging; use of a centralized repository (SourceForge) for code, executable and data sharing;  and use of the CMake/CTest/CDash  suite for cross-platform  development,  testing, and automatic builds. Virtual machines with different versions of Windows, MacOS and Linux operating systems generate nightly builds and execute test code, uploading a binary to the central SourceForge repository. ANTs and ITK-SNAP are documented through video and text tutorials, housed online on dedicated websites [11, 88]. A similar infrastructure will be developed for the software resources proposed in Aim 1.

3(c.3) Specific Aim 2.  Validate and disseminate the developed ITK-Lung resources by leveraging use cases from a broad network of partner investigators representing the state-of-the-science in lung imaging research

This aim builds on the project team’s long and successful track record of collaboration with the general user community.  In particular, the investigator-driven  studies presented below are carefully selected both for their capacity to fully exercise the developed tools and to provide a comprehensive representation of the various processing and analysis tasks of interest to the community. Two evaluation groups will serve as primary and secondary beta testers, respectively: institutional and external collaborators.  For the former, the development team will be embedded in the labs of project co-investigators, providing training to lab personnel and assisting with the creation of customized analysis pipelines.  These experiences will be used to develop tutorial materials and processing examples that our external collaborators will independently test by processing their own data in-house with beta versions of the proposed software and data resources.  Similarly, the dissemination and user support mechanisms outlined in the Resource Sharing Plan will be refined by employing these as much as possible to communicate and interact with the external collaborator group.

Institutional collaborators

3(c.3.1) Novel imaging biomarkers for Chronic Obstructive Pulmonary Disease (COPD). Co-investigator Mike Shim and his group have been actively developing 3D hyperpolarized xenon-129 dissolved-phase MRI (HXe MRI) as a sen- sitive biomarker for accurately characterizing phenotypes and severity of COPD. This protocol permits regional mapping of ventilation and gas uptake by tissue and blood in human lungs with single breath hold [89–91]. This project plans to establish connectivity between these advanced HXe MRI imaging signatures and important clinical outcomes of COPD to advance HXe MRI as a novel clinical diagnostic tool.  This new biomarker tool will naturally lead to deeper mechanistic understanding of COPD at the molecular-physiologic  and clinical levels and support identification of potential pathophysiologic derangement associated with COPD and a new method to accurately predict therapeutic response to current standard COPD therapies. Re- finement of HXe MRI as a pulmonary diagnostic tool is anticipated to encourage development of new clinical interventions.

HXe MRI is the first non-invasive imaging technique that can provide regional information about three unique characteristics of lung function: lung ventilation, size and connectedness of distal alveolar airspaces, and HXe gas transfer from airspaces to red blood cells. HXe MRI, therefore, is anticipated to overcome the limitation of pulmonary function testing (PFT) which only provides physiologic parameters of the lung as a whole unit, and High Resolution CT (HRCT) which only provides anatomic characterization without physiologic information.  HXe MRI has potential to detect pathologic changes present in COPD pa- tients with high sensitivity and specificity previously unattainable by the current clinical standard (PFT and HRCT). Moreover, HXe MRI can determine whether gas transfer abnormalities are due to impaired ventilation or reduced gas-exchange, and thus provide new insights into the pathogenesis of COPD in individual patients.

Crucial to the success of establishing the utility of HXe MRI as a sensitive biomarker for accurately characterizing COPD phe- notypes is quantification of imaging signatures in an automated and robust fashion.  Identification of ventilation dead space (VD ) for correlation with GOLD classification will utilize the ventilation-based segmentation functionality in ITK-Lung [38]. In order to determine lobar values of HXe MRI, this study will utilize the recently proposed lobar estimation algorithm [50] that will be available for both proton MRI and CT.

3(c.3.2) Hyperpolarized gas imaging in children with asthma. Advances in rapid image sequencing methods have fa- cilitated the acquisition of high-quality hyperpolarized gas MR images in pre-school children [92]. Furthermore, improvements in image processing and signal intensity analysis have made possible accurate measurements of lung volume compartments [38].  Co-investigators Gerry Teague and Talissa Altes are applying these innovations in children with asthma to study whether the lung defect volume % as measured by hyperpolarized lung MRI correlates with a range of clinical features.  They hypothesize the ventilation defect volume % would be higher in children with severe asthma, and correlate not only with the degree of airflow limitation, but indicators of asthma control, treatment, and inflammation.

Precise measurement of ventilation volumes by hyperpolarized noble gas MRI not only has the potential to resolve the spatial and temporal characteristics of gas distribution in children with asthma, but could also expand clinically relevant information in regards to asthma severity and its features. In the past, simple computer-assisted systems [93] or hand counts of visual defects were used to estimate the ventilation defect volume [94]. Development of more advanced techniques (in terms of acquisition and analysis) will facilitate rapid conversion of complex hyperpolarized gas signal data into volume compartments for clinical applications.

Absolutely crucial to the advanced techniques being developed by Dr. Teague and his group are sophisticated image analysis

tools like the ones being proposed in this project.  For example, our ventilation-based segmentation method is already being used to determine volumetric compartments based on lung function.  Additional “cleaning” necessary for these data include denoising techniques [51] implemented and made available in ANTs. Lobe estimation will be possible by refining the techniques originally described in [50].

3(c.3.3) Characterization of COPDGene cohort by hyperpolarized gas (HP) MRI. Co-investigator Rahim Rizi is leading a study of lung function and structure in COPD using HP MRI. Once inhaled, this gas can tell the researcher how well specific lung regions replace the air during the normal breathing cycle (Fractional Ventilation, FV), how much oxygen is in the airspaces (Oxygen Tension, PAO2), and if the normal spongy tissue structure has been compromised in lung disease (Apparent Diffusion Coefficient). Subjects will include those at risk for lung disease, and those displaying mild and moderate COPD. They will be mostly drawn from the well-characterized population currently enrolled in the COPDGene trial (10,000 subjects overall) such that standard clinical images (End Inspiration and End Expiration CT) and Pulmonary Function Tests (PFTs), as well as genetic sequencing, will already have been done. Each subject will be imaged twice during the course of the five-year study, and regional features will be compared between the CT and MRI images to the genetic markers, changes in clinical measurements, and patient quality of life.

The proposed study will generate non-invasive biomarkers of COPD progression derived from minute, short-term alterations in lung function and microstructure. Due to the excellent safety profile of MRI, these metrics will be appropriate for use in novel, flexible study designs.  Perhaps most importantly, this research will enhance understanding of the natural history of COPD. In doing so, it will provide a vital supplement to ongoing efforts to identify COPD subtypes by adding substantial physiologic detail to descriptions of this disease. The overall goals of the study experiments are: a) to develop imaging markers that better identify early COPD; b) to develop tests that predict health deterioration due to COPD; c) to determine if specific patterns of disease progression are associated with genetic markers identified in the larger COPDGene study; and d) to determine if disease progression is in part caused by excessive stretch in regions of the lung next to blocked-off areas unable to inflate normally.

More than 200 million people suffer from COPD worldwide. Yet effectively assessing the progression of this increasingly preva- lent disease and monitoring its response to treatment remain problematic. Hyperpolarized gas MRI can help rectify these issues by providing sensitive measurements of lung physiology and microstructure, but its adoption by clinicians and investigators has been slow. In contrast, CT-based methods for measuring emphysema, airway wall thickening, and expiratory air trapping have become common in COPD clinical studies. There are several reasons for this: CT is more accessible, its images possess ex- cellent spatial resolution, and quantification of these images is currently superior. However, most CT-based parameters have only an indirect relation to physiology, and the modality exposes patients to ionizing radiation.  Both of these shortcomings can be addressed by HP gas MRI. Consequently, the study seeks to more fully exploit the clinical potential of HP gas MRI by optimizing and testing parameters for the regional assessment of COPD patients and symptomatic smokers.

A novel multi-breath HP MRI technique allows for the simultaneous measurement of fractional ventilation (FV), regional partial pressure of oxygen (PAO2), and apparent diffusion coefficient (ADC). Obtaining all three parameters in a single scan reduces the necessary amount of imaging gas while increasing accuracy by correcting artifacts associated with collateral ventilation and the slow filling of parenchyma in diseased lungs.  Each of these metrics allows for the investigation of a vital aspect of lung disease progression and their comparison with the current CT-based standard of care will help to more clearly understand different features and phenotypes of COPD.

The proposed image analysis software will be central to the successful conduct of the following tasks necessary to establish the goals of this study:

•  Registration of the multibreath/multislice  gas MRI images of the whole lung consisting of a minimum of seven time points

•  Registration and analysis of inspiratory and expiratory CT for airway changes to assess airway collapsibility and remod- eling and other CT markers

•  Registration and analysis of inspiratory and expiratory CT with MRI to study the similarities and differences of the two modalities in phenotyping the COPD population

•  Registration of the follow-up MRI and CT images (two years) to determine if disease progression is in part caused by excessive stretch in regions of the lung next to blocked-off areas unable to inflate normally (based on the baseline MRI and CT)

3(c.3.4) Advanced image analysis of CT for early diagnosis and prognosis of bronchiolitis obliterans syndrome (BOS) in lung transplant patients.  Co-investigators  Eduardo Barbosa and Warren Gefter are conducting a retro- spective study of more than 300 lung transplant patients to advance the early diagnosis of BOS. Lung transplantation is an established treatment for end-stage, irreversible pulmonary disease, particularly due to COPD and interstitial lung disease (ILD). While continued improvements in surgical techniques and immunosuppressive medications have reduced the compli- cation rates and increased short-term survival after the procedure, chronic allograft rejection due to bronchiolitis obliterans (a fibrous obliterative disease of bronchioles representing the histological hallmark of chronic rejection and resulting in obstruc- tive pulmonary physiology) remains the major cause of morbidity and mortality after six months following transplantation.

Bronchiolitis obliterans currently represents the greatest limitation to long-term survival after lung transplantation.   While the diagnosis of bronchiolitis obliterans is a pathologic one and therefore requires invasive biopsy, the distribution of disease is patchy, with focal areas of abnormality surrounded by normal lung, and consequently even biopsies may fail to demonstrate the diagnosis. For these reasons, the International Society for Heart and Lung Transplantation has recommended using declining spirometry, termed bronchiolitis obliterans syndrome (BOS), as a surrogate marker of chronic allograft rejection.  In clinical practice, the diagnosis of BOS is suspected based on an unexplained decline in lung function (measured by PFT, of greater than

20% of baseline FEV1) and worsening cough and dyspnea, in the absence of other explanations such as pulmonary infection or congestive heart failure. MDCT plays an important role by demonstrating low attenuation areas representing air trapping, particularly on expiratory images, which correlate with the presence of bronchiolitis obliterans. Prior studies reported limited sensitivity for the early diagnosis of bronchiolitis obliterans; however these utilized semi-quantitative or qualitative assessment of air trapping in non-volumetric data sets. This study aims to assess whether more advanced, fully automated imaging analysis can detect early BOS prior to development of clinically apparent disease.

PFT is the current reference standard for diagnosis of BOS; however, by the time PFT abnormalities beyond the threshold of BOS diagnosis ensue, the disease is already manifest and is not reversible with existing therapies. It is conceivable that sophisticated analysis of CT, including quantitative attenuation masks in inspiratory and expiratory datasets, image registration, and texture based feature extraction may allow earlier detection of BOS in the preclinical phase, potentially generating surrogate biomarkers for drug trials and earlier prognostication.

Application of the proposed advanced software tools and algorithms in this project for quantitative analysis of CT images in lung transplantation patients will be crucial to enable computation of an array of first and second order statistics that would capture not only attenuation maps but also regional deformation and texture based features. In combination, this would allow multiparametric statistical modeling that may predict which patients will develop BOS before PFT abnormalities beyond the diagnostic threshold ensue. Such tools will be extended to other diffuse lung diseases, potentially generating new biomarkers for diagnosis, prognostication and therapeutic trials.

Novel PET/CT biomarkers for early prediction of treatment response and survival in patients with non small cell lung cancer (NSCLC).  PET and CT have traditionally played important roles in diagnosis, staging, and predicting treatment response in NSCLC [3-5]. Although many studies have demonstrated diagnostic and prognostic value of tumor metabolic burden and morphological information derived from PET/CT imaging measures [5, 6], current ability to predict treatment response and survival following radiation therapy in NSCLC is far from satisfactory [7]. The poor performance of existing methods may be due to a number of reasons: first, most studies have been limited to primary tumor lesions and ignored the information available in other lesions/regions [8]; second, many investigations have focused on global image measures of tumors, such as those derived from standardized uptake value (SUV) of PET images and tumor morphological information, which may not be sufficient to capture subtleties of tumor characteristics and tumor heterogeneity [9, 10]; and finally, group-level statistical analysis strategies are typically adopted to evaluate imaging markers, which may not be effective for personalized prediction [11].

To improve prediction of radiation therapy treatment response and survival in patients with NSCLC based on PET/CT imaging data, co-investigator David Mankoff and collaborators are developing methods to adaptively extract image features from the data that are then input into a multi-task learning framework constructed to jointly model (and leverage potential statistical correlations between) the problems of predicting treatment response and survival.  The proposed software tools will fulfill essential pre-processing needs of the study: PET/CT data will be co-registered and a wealth of image features calculated using ITK-Lung.

3.	Lardinois, D., W. Weder, T.F. Hany, E.M. Kamel, S. Korom, B. Seifert, G.K. von Schulthess, and H.C. Steinert, Staging of non-small-cell lung cancer with integrated positron-emission tomography and computed tomography. N Engl J Med, 2003. 348(25): p. 2500-7.

4.	Lee, H.Y., H.J. Lee, Y.T. Kim, C.H. Kang, B.G. Jang, D.H. Chung, J.M. Goo, C.M. Park, C.H. Lee, and K.W. Kang, Value of combined interpretation of computed tomography response and positron emission tomography response for prediction of prognosis after neoadjuvant chemotherapy in non-small cell lung cancer. J Thorac Oncol, 2010. 5(4): p. 497-503.

5.	Horne, Z.D., D.A. Clump, J.A. Vargo, S. Shah, S. Beriwal, S.A. Burton, A.E. Quinn, M.J. Schuchert, R.J. Landreneau, N.A. Christie, J.D. Luketich, and D.E. Heron, Pretreatment SUVmax predicts progression-free survival in early-stage non-small cell lung cancer treated with stereotactic body radiation therapy. Radiat Oncol, 2014. 9: p. 41.

6.	Zhang, H., K. Wroblewski, S. Liao, R. Kampalath, B.C. Penney, Y. Zhang, and Y.L. Pu, Prognostic Value of Metabolic Tumor Burden from F-18-FDG PET in Surgical Patients with Non-small-cell Lung Cancer. Academic Radiology, 2013. 20(1): p. 32-40.

7.	Berman, A.T., S.S. Ellenberg, and C.B. Simone, Predicting Survival in Non–Small-Cell Lung Cancer Using Positron Emission Tomography: Several Conclusions From Multiple Comparisons. Journal of Clinical Oncology, 2014. 32(15): p. 1631-1632.

8.	Ryu, J.S. and I.Y. Hyun, Prognostic impact of [18F]fluorodeoxyglucose positron emission tomography scanning in the era of molecular oncology. J Clin Oncol, 2014. 32(15): p. 1630.

9.	Chicklore, S., V. Goh, M. Siddique, A. Roy, P.K. Marsden, and G.J. Cook, Quantifying tumour heterogeneity in 18F-FDG PET/CT imaging by texture analysis. Eur J Nucl Med Mol Imaging, 2013. 40(1): p. 133-40.

10.	Depeursinge, A., A. Foncubierta-Rodriguez, D. Van De Ville, and H. Muller, Three-dimensional solid texture analysis in biomedical imaging: review and opportunities. Med Image Anal, 2014. 18(1): p. 176-96.

11.	Henderson, R. and N. Keiding, Individual survival time prediction using statistical models. Journal of Medical Ethics, 2005. 31(12): p. 703-706.

External collaborators

3(c.3.5) Comparison of automated multi-modality registration methodologies to manual registration for as- sessing lung CT bronchial morphologic changes and hyperpolarized helium MR ventilation defects in asthma patients: Can automation speed the work flow for combining structure and function using airways measures from CT and ventilation measures from HP gas MRI? Our collaborators Sean Fain and Mark Schiebler, University of Wisconsin, are part of the SARP (Severe Asthma Research Program) team developing imaging biomarkers of asthma sever- ity for predicting asthma exacerbation. The approach of finding airway abnormalities that correlate with ventilation defects is viable only with the availability of robust image registration across the two modalities.  Furthermore, translation to the clinic will require standardized implementations across sites, and the project’s open-source platform is ideal for this purpose.

3(c.3.6) Validation of voxel-based ventilation CT. Our collaborator Jim Wild, University of Sheffield, has been active in the field of methods development for quantitative pulmonary imaging and its clinical translation for more than a decade.  Relevant to this project, his group requires advanced registration capabilities to validate a novel CT technique for obtaining high-resolution images of pulmonary ventilation.  In addition, there is need within his research for multi-modality registration of pulmonary CT and MRI images as well as segmentation of key lung structures as part of standard processing workflows.

3(c.3.7) Deep functional phenotyping of COPD. Our collaborator Hans-Ulrich Kauczor, University Medical Cen- ter Heidelberg,  is leading the COSYCONET (German COPD and Systemic Consequences–Comorbidities  Network) study, the world’s first prospective multicenter trial comparing proton MRI and CT imaging for characterizing COPD, with the latter modality serving as the reference standard.  Automated image registration and segmentation will play a vital role in defining the quantitative CT (air trapping, airway collapsibility and remodeling, and pulmonary blood volume and vascular pruning) and MR (air trapping, perfusion volume defects, and hypoxic vasoconstriction) imaging biomarkers that form the basis for the study.

3(c.3.8) Longitudinal imaging follow-up in COPD and lung cancer. Our collaborator Joon-Beom Seo, Asan Medi- cal Center, directs the imaging component of the Korean Obstructive Lung Disease (KOLD) cohort study, which has collected over 1000 COPD cases from 17 participating centers with repeated imaging since 2005.  He also leads a national lung cancer radiomics project that has accrued 800 cases to date. In both studies, robust image registration is essential to tracking changes over time, and segmentation is an additional requirement to support automated lesion delineation for the cancer project.

3(c.3.9) Advanced image processing pipelines for MR image-guided pulmonary therapy decisions and sup- port. Our collaborator Grace Parraga, University of Western Ontario, has been at the forefront of MR imaging of lung structure and function since 2005.  A major challenge hampering widespread translation of current pulmonary imaging ad- vances is the lack of precision in their interpretation, thereby complicating the planning and guiding of targeted therapies. The project’s software tools will enable the development of robust analysis pipelines for the translation of in vivo imaging biomark-

ers in an open and consistent manner across platforms and centers.  This work, carried out in collaboration with industrial partners, will support patient phenotyping and stratification to therapy as well as measurement of longitudinal changes and response to therapy.

3(c.3.10) Functional MR imaging of the lungs using hyperpolarized and inert gases.  Our collaborator Mitchell Albert, Thunder Bay Regional Research Institute, has been advancing the use of inert fluorinated gases that can be breathed continuously in order to measure indicators of wash-in, wash-out and air trapping with dynamic imaging protocols. To compute the wash-in and wash-out time constants on a pixelwise basis, access to accurate and reliable image registration tools will be essential.

3(c.3.11) Multi-modality imaging studies of pulmonary diseases. Our collaborator Edwin van Beek, University of Edinburgh, has been conducting various multimodality studies, including the evaluation of pulmonary fibrosis using both gadolinium-enhanced  MRI and contrast-enhanced CT perfusion imaging, and the assessment of lung nodules using PET-CT and CT perfusion imaging. These relatively new techniques would benefit from quantitative analysis of contrast enhancement, and advanced image registration and segmentation capabilities will both be necessary toward this end.

3(c.3.12) Computational imaging biomarkers for diverse thoracic malignancies.  Our collaborator Yoshiharu Ohno, Kobe University, leads a comprehensive program of multi-modality imaging (CT, MRI, PET, molecular imaging) research on lung cancer, COPD, ILD, and pulmonary infectious diseases. His myriad image analysis needs include quantitative characterization of: lung parenchyma and airway changes; regional perfusion, ventilation, and metabolism from whole-lung perfusion and multi-modality metabolic imaging studies; ultra-short TE MRI data; and regional and global kinematics.

More details about the research, data, and advances enabled by the proposed software tools for each of the extramural partner studies above can be found in the corresponding letter of support.  The nature and diversity of the imaging data collected for these studies will be a stringent test of the ease of use, interactivity, and flexibility of the developed processing and analysis software resources in this project. Moreover, the studies will yield valuable additions to the portfolio of use cases that serve as primary reference and instructional material for the software.

3(c.3.13) Sub-Aim 2a will disseminate the results of the project through open-source publication of the code, annotated processed data, online user support, and conduct of hands-on training workshops. ITK is the leading open-source development system for medical image analysis, and in recognition of this project’s value to the field, ITK will lend its infrastructure to provide long-term hosting services for the developed resources as well as incorporate ITK-Lung training into its educational programs that are offered in conjunction with major scientific (e.g., annual International Conference on Medical Image Computing and Computer Assisted Intervention) and user forums (e.g., hackathons); see Yoo letter of support. Further leveraging of ITK support will include formalized advisory input from its core development team (of which the project team is a member), and access to and promotion within its extensive outreach program.  Complete dissemination details can be found in the Resource Sharing Plan.

3(c.4) Risks and alternatives

Lung imaging analysis is made more difficult by the inherent challenges posed by the dynamic nature of the lung environment in acquiring high quality images.  This is compounded by the wider range of imaging signal characteristics in the chest due to the appearance of air, fat, soft tissue and bone, together with their interfaces, in the images.  The complexity of lung images and diversity of protocols and equipment/modalities used for their acquisition, therefore, demand flexible and tunable (i.e., open and programmable) tools, with manifold capabilities carefully curated to cover essential analysis and processing tasks, all of which ideally integrated within a single coherent toolbox – this is precisely the overall goal and deliverable of the proposed project. While the proposed infrastructure is complex and integrates multiple cutting-edge technologies, we do not anticipate significant problems in its development and consider the risk of failure of the project to be very low. Our optimism is based on the extensive preliminary work that has been performed over a significant period of time to successfully demonstrate feasibility of every aspect of the project. Given the level of expertise and experience of our interdisciplinary team and the well-defined scope of the software development and engineering problems, we are highly confident in a successful outcome.

3(c.5) Timeline

Aim 1:  Software development will take place in Years 1-5, with Year 1 focused on refactoring of existing ANTs-based code and integration with ITK; Year 2 focused on incorporation of new methods to support expanded functionality beyond core algorithms; Year 3 focused on ITK-SNAP-based GUI implementation;  Year 4 focused on releasing a fully functional system; and Year 5 focused on incremental improvements based on Aim 2 studies. Data collection and annotation (lung and lobes) for the multi-atlas libraries will take place in Years 1-2, followed by segmentations (vessels and airways) and template building in Year 3. Aim 2: A preliminary version of the software will be deployed at evaluation sites toward the end of Year 2 (with quarterly software updates thereafter), and testing will run through Year 4. Documentation and dissemination efforts will take place throughout the course of the project.

References

1.  Wang, Y.-X. and Deng, M. “Medical Imaging in New Drug Clinical Development”  J Thorac Dis 2, no.  4 (2010):

245–52. doi:10.3978/j.issn.2072-1439.2010.11.10

2. Zhao, B., Tan, Y., Bell, D. J., Marley, S. E., Guo, P., Mann, H., Scott, M. L. J., Schwartz, L. H., and Ghiorghiu, D. C. “Exploring Intra- and Inter-Reader Variability in Uni-Dimensional,  Bi-Dimensional,  and Volumetric Measurements of Solid Tumors on CT Scans Reconstructed at Different Slice Intervals” Eur J Radiol 82, no.  6 (2013):  959–68. doi:10.1016/j.ejrad.2013.02.018

3.   McErlean,  A., Panicek,  D. M., Zabor, E. C., Moskowitz,  C. S., Bitar,  R., Motzer, R. J., Hricak, H., and Ginsberg,  M. S. “Intra- and Interobserver Variability in CT Measurements in Oncology” Radiology 269, no.  2 (2013):  451–9. doi:10.1148/radiol.13122665

4. Fischl, B. “FreeSurfer” Neuroimage 62, no. 2 (2012): 774–81. doi:10.1016/j.neuroimage.2012.01.021

5. Jenkinson, M., Beckmann, C. F., Behrens, T. E. J., Woolrich, M. W., and Smith, S. M. “FSL” Neuroimage 62, no. 2 (2012):

782–90. doi:10.1016/j.neuroimage.2011.09.015

6. Cox, R. W. “AFNI: What a Long Strange Trip It’s Been” Neuroimage 62, no. 2 (2012): 743–7. doi:10.1016/j.neuroimage.2011.08

7. Ashburner, J. “SPM: A History” Neuroimage 62, no. 2 (2012): 791–800. doi:10.1016/j.neuroimage.2011.10.025

8. Hoffman, E. A., Lynch, D. A., Barr, R. G., Beek, E. J. R. van, Parraga, G., and IWPFI Investigators.  “Pulmonary CT and MRI Phenotypes That Help Explain Chronic Pulmonary Obstruction Disease Pathophysiology and Outcomes” J Magn Reson Imaging (2015): doi:10.1002/jmri.25010

9. Yunwen, Y. and Kishida, K. “Toward an Understanding of the Motivation of Open Source Software Developers”

Software engineering, 2003. proceedings. 25th international conference on (2003): 419–429. doi:10.1109/ICSE.2003.1201220

10. (2008): Available at http://fsmsh.com/2845

11. Available at http://picsl.upenn.edu/software/ants/

12. Tustison, N. J. and Avants, B. B. “Explicit B-Spline Regularization in Diffeomorphic Image Registration” Front

Neuroinform 7, (2013): 39. doi:10.3389/fninf.2013.00039

13. Tustison, N. J., Cook, P. A., Klein, A., Song, G., Das, S. R., Duda, J. T., Kandel, B. M., Strien, N. van, Stone, J. R., Gee, J. C., and Avants, B. B. “Large-Scale Evaluation of ANTs and FreeSurfer Cortical Thickness Measurements” Neuroimage

99, (2014): 166–79. doi:10.1016/j.neuroimage.2014.05.044

14. Bajcsy, R. and Broit, C. “Matching of Deformed Images” Sixth international conferenceon pattern recognition (iCPR’82)

(1982): 351–353.

15.  Bajcsy, R. and Kovacic, S. “Multiresolution Elastic Matching” Computer Vision, Graphics, and Image Processing 46, no. 1 (1989): 1–21. doi:10.1016/S0734-189X(89)80014-3, Available at http://dx.doi.org/10.1016/S0734-189X(89)80014-3

16.  Gee, J. C., Reivich, M., and Bajcsy, R. “Elastically Deforming 3D Atlas to Match Anatomical Brain Images” J Comput Assist Tomogr 17, no. 2 (): 225–36.

17. Murphy, K., Ginneken, B. van, Reinhardt, J. M., Kabus, S., Ding, K., Deng, X., Cao, K., Du, K., Christensen, G. E., Garcia, V., Vercauteren, T., Ayache, N., Commowick, O., Malandain, G., Glocker, B., Paragios, N., Navab, N., Gorbunova, V., Sporring, J., Bruijne, M. de, Han, X., Heinrich, M. P., Schnabel, J. A., Jenkinson, M., Lorenz, C., Modat, M., McClelland, J. R., Ourselin, S., Muenzing,  S. E. A., Viergever,  M. A., De Nigris, D., Collins, D. L., Arbel, T., Peroni, M., Li, R., Sharp, G. C., Schmidt- Richberg, A., Ehrhardt, J., Werner, R., Smeets, D., Loeckx, D., Song, G., Tustison, N., Avants, B., Gee, J. C., Staring, M., Klein, S., Stoel, B. C., Urschler, M., Werlberger, M., Vandemeulebroucke,  J., Rit, S., Sarrut, D., and Pluim, J. P. W. “Evaluation of Registration Methods on Thoracic CT: The EMPIRE10 Challenge” IEEE Trans Med Imaging 30, no.  11 (2011):

1901–20. doi:10.1109/TMI.2011.2158349

18. Klein, A., Andersson, J., Ardekani, B. A., Ashburner, J., Avants, B., Chiang, M.-C., Christensen, G. E., Collins, D. L., Gee, J., Hellier, P., Song, J. H., Jenkinson, M., Lepage, C., Rueckert, D., Thompson, P., Vercauteren, T., Woods, R. P., Mann, J. J., and Parsey, R. V. “Evaluation of 14 Nonlinear Deformation Algorithms Applied to Human Brain MRI Registration” Neuroimage 46, no. 3 (2009): 786–802. doi:10.1016/j.neuroimage.2008.12.037

19. Tustison, N. J., Yang, Y., and Salerno, M. “Advanced Normalization Tools for Cardiac Motion Correction” Statis- tical atlases and computational models of the heart - imaging and modelling challenges 8896, (2015): 3–12. doi:10.1007/978-

3-319-14678-2_1, Available at http://dx.doi.org/10.1007/978-3-319-14678-2_1

20.  Avants, B. B., Epstein, C. L., Grossman, M., and Gee, J. C. “Symmetric Diffeomorphic Image Registration with

Cross-Correlation:  Evaluating Automated Labeling of Elderly and Neurodegenerative  Brain” Med Image Anal

12, no. 1 (2008): 26–41. doi:10.1016/j.media.2007.06.004

21. Tustison, N. J., Song, G., Gee, James C, and Avants, B. B. “Two Greedy SyN Variants for Pulmonary Image Regis-

tration” Evaluation of methods for pulmonary image registration (EMPIRE10) (2012):

22.    Avants,  B.  B.,  Yushkevich,  P.,  Pluta,  J.,  Minkoff,  D.,  Korczykowski,  M.,  Detre,  J.,  and  Gee,  J.  C.  “The  Optimal Template Effect in Hippocampus Studies of Diseased Populations” Neuroimage 49, no.   3 (2010):  2457–66. doi:10.1016/j.neuroimage.2009.09.062

23. Tustison, N. J., Shrinidhi, K. L., Wintermark, M., Durst, C. R., Kandel, B. M., Gee, J. C., Grossman, M. C., and Avants, B. B. “Optimal Symmetric Multimodal Templates and Concatenated Random Forests for Supervised Brain Tumor Segmentation (Simplified) with AN T sR”  Neuroinformatics (2014): doi:10.1007/s12021-014-9245-2

24.  Avants, B. B., Duda, J. T., Kilroy, E., Krasileva, K., Jann, K., Kandel, B. T., Tustison, N. J., Yan, L., Jog, M., Smith, R., Wang, Y., Dapretto, M., and Wang, D. J. J. “The Pediatric Template of Brain Perfusion” Sci Data 2, (2015):  150003. doi:10.1038/sdata.2015.3

25.  Datta, R., Lee, J., Duda, J., Avants, B. B., Vite, C. H., Tseng, B., Gee, J. C., Aguirre, G. D., and Aguirre, G. K. “A Digital

Atlas of the Dog Brain” PLoS One 7, no. 12 (2012): e52140. doi:10.1371/journal.pone.0052140

26.   McMillan,  C. T.,  Avants,  B. B.,  Cook,  P.,  Ungar,  L.,  Trojanowski,  J. Q.,  and Grossman,  M. “The Power  of Neu- roimaging Biomarkers for Screening Frontotemporal Dementia” Hum Brain Mapp 35, no.  9 (2014):  4827–40. doi:10.1002/hbm.22515

27. Cook, P. A., McMillan, C. T., Avants, B. B., Peelle, J. E., Gee, J. C., and Grossman, M. “Relating Brain Anatomy and Cog-

nitive Ability Using a Multivariate Multimodal Framework” Neuroimage 99, (2014): 477–86. doi:10.1016/j.neuroimage.2014.0

28. Tustison, N. J., Avants, B. B., Cook, P. A., Kim, J., Whyte, J., Gee, J. C., and Stone, J. R. “Logical Circularity in Voxel- Based Analysis:  Normalization Strategy May Induce Statistical Bias” Hum Brain Mapp 35, no.  3 (2014):  745–59. doi:10.1002/hbm.22211

29.  Tustison, N. J., Contrella, B., Altes, T. A., Avants, B. B., Lange, E. E. de, and Mugler, J. P. “Longitudinal Assessment of Treatment Effects on Pulmonary Ventilation Using 1H/3He MRI Multivariate Templates” Proc.  sPIE 8672, medical imaging 2013: Biomedical applications in molecular, structural, and functional imaging (2013):

30.  Vannier, M. W., Butterfield, R. L., Jordan, D., Murphy, W. A., Levitt, R. G., and Gado, M. “Multispectral Analysis of

Magnetic Resonance Images” Radiology 154, no. 1 (1985): 221–4. doi:10.1148/radiology.154.1.3964938

31. Dempster, A., Laird, N., and Rubin, D. “Maximum Likelihood Estimation from Incomplete Data Using the EM Algorithms” Journal of the Royal Statistical Society 39, (1977): 1–38.

32.  Cline, H. E., Lorensen, W. E., Kikinis, R., and Jolesz, F. “Three-Dimensional  Segmentation of MR Images of the

Head Using Probability and Connectivity” J Comput Assist Tomogr 14, no. 6 (): 1037–45.

33. Kikinis, R., Shenton, M. E., Gerig, G., Martin, J., Anderson, M., Metcalf, D., Guttmann, C. R., McCarley, R. W., Lorensen, W., and Cline, H. “Routine Quantitative Analysis of Brain and Cerebrospinal Fluid Spaces with MR Imaging” J Magn Reson Imaging 2, no. 6 (): 619–29.

34.  Geman, S. and Geman, D. “Stochastic Relaxation, Gibbs Distributions, and the Bayesian Restoration of Im- ages” IEEE Trans Pattern Anal Mach Intell 6, no. 6 (1984): 721–41.

35. Held, K., Rota Kops, E., Krause, B. J., Wells, W. M., 3rd, Kikinis, R., and Müller-Gärtner, H. W. “Markov Random Field

Segmentation of Brain MR Images” IEEE Trans Med Imaging 16, no. 6 (1997): 878–86. doi:10.1109/42.650883

36.  Van Leemput, K., Maes, F., Vandermeulen, D., and Suetens, P. “Automated Model-Based Tissue Classification of

MR Images of the Brain” IEEE Trans Med Imaging 18, no. 10 (1999): 897–908. doi:10.1109/42.811270

37. Ashburner, J. and Friston, K. J. “Unified Segmentation” Neuroimage 26, no. 3 (2005): 839–51. doi:10.1016/j.neuroimage.2005.

38.  Tustison, N. J., Avants, B. B., Flors, L., Altes, T. A., Lange, E. E. de, Mugler, J. P., 3rd, and Gee, J. C. “Ventilation- Based Segmentation of the Lungs Using Hyperpolarized (3)He MRI” J Magn Reson Imaging 34, no. 4 (2011): 831–41. doi:10.1002/jmri.22738

39. Avants, B. B., Tustison, N. J., Wu, J., Cook, P. A., and Gee, J. C. “An Open Source Multivariate Framework for n- Tissue Segmentation with Evaluation on Public Data” Neuroinformatics 9, no. 4 (2011): 381–400. doi:10.1007/s12021-

011-9109-y

40.  Altes, T., Johnson, M., III, J. M., Miller, G. W., Flors, L., Mata, J., Salinas, C., Tustison, N., Lee, P.-S., Song, T., Froh, K. Y. D., and Botfield, M. “The Effect of Ivacaftor, an Investigational CFTR Potentiator, on Hyperpolarized No- ble Gas Magnetic Resonance Imaging in Subjects with Cystic Fibrosis Who Have the G551D-CFTR Mutation” PATHOGENESIS AND CLINICAL ISSUES IN CYSTIC FIBROSIS B35, (2012): A2814–A2814.

41.  Teague, W. G., Tustison, N. J., and Altes, T. A. “Ventilation Heterogeneity in Asthma” J Asthma 51, no.  7 (2014):

677–84. doi:10.3109/02770903.2014.914535

42.  Kirby, M., Pike, D., Coxson, H. O., McCormack, D. G., and Parraga, G. “Hyperpolarized (3)He Ventilation Defects

Used to Predict Pulmonary Exacerbations in Mild to Moderate Chronic Obstructive Pulmonary Disease” Radi- ology 273, no. 3 (2014): 887–96. doi:10.1148/radiol.14140161

43.  Sled, J. G., Zijdenbos, A. P., and Evans, A. C. “A Nonparametric Method for Automatic Correction of Intensity

Nonuniformity in MRI Data” IEEE Trans Med Imaging 17, no. 1 (1998): 87–97. doi:10.1109/42.668698

44. Boyes, R. G., Gunter, J. L., Frost, C., Janke, A. L., Yeatman, T., Hill, D. L. G., Bernstein, M. A., Thompson, P. M., Weiner, M. W., Schuff, N., Alexander, G. E., Killiany, R. J., DeCarli, C., Jack, C. R., Fox, N. C., and ADNI Study.  “Intensity Non- Uniformity Correction Using N3 on 3-T Scanners with Multichannel Phased Array Coils” Neuroimage 39, no. 4 (2008): 1752–62. doi:10.1016/j.neuroimage.2007.10.026

45.   Tustison,  N. J.,  Awate,  S. P.,  Cai,  J.,  Altes,  T. A.,  Miller,  G. W.,  Lange,  E. E. de,  Mugler,  J. P.,  3rd,  and Gee,  J. C. “Pulmonary  Kinematics  from Tagged Hyperpolarized  Helium-3  MRI” J Magn Reson Imaging 31, no.   5 (2010):

1236–41. doi:10.1002/jmri.22137

46. Wang, H. and Yushkevich, P. A. “Multi-Atlas Segmentation with Joint Label Fusion and Corrective Learning-an

Open Source Implementation” Front Neuroinform 7, (2013): 27. doi:10.3389/fninf.2013.00027

47. Wang, H., Suh, J. W., Das, S. R., Pluta, J. B., Craige, C., and Yushkevich, P. A. “Multi-Atlas Segmentation with Joint

Label Fusion” IEEE Trans Pattern Anal Mach Intell 35, no. 3 (2013): 611–23. doi:10.1109/TPAMI.2012.143

48.  Yushkevich, P. A., Wang, H., Pluta, J., Das, S. R., Craige, C., Avants, B. B., Weiner, M. W., and Mueller, S. “Nearly Automatic Segmentation of Hippocampal Subfields in in Vivo Focal T2-Weighted MRI” Neuroimage 53, no.  4 (2010): 1208–24. doi:10.1016/j.neuroimage.2010.06.040

49. Available at https://masi.vuse.vanderbilt.edu/workshop2013/index.php/Main_Page

50. Tustison, N. J., Qing, K., Wang, C., Altes, T. A., and Mugler, J. P., 3rd. “Atlas-Based Estimation of Lung and Lobar

Anatomy in Proton MRI” Magn Reson Med (Accepted):

51.    Manjón,  J.  V.,  Coupé,  P.,  Martí-Bonmatí,  L.,  Collins,  D.  L.,  and  Robles,  M.  “Adaptive  Non-Local  Means  De- noising of MR Images with Spatially Varying Noise Levels” J Magn Reson Imaging 31, no.  1 (2010):  192–203. doi:10.1002/jmri.22003

52.  Wang, H., Suh, J. W., Das, S. R., Pluta, J., Craige, C., and Yushkevich, P. A. “Multi-Atlas Segmentation with Joint

Label Fusion” IEEE Trans Pattern Anal Mach Intell (2012): doi:10.1109/TPAMI.2012.143

53. “MICCAI 2012 Workshop on Multi- Atlas Labeling” (2012):

54.  Asman, A., Akhondi-Asl, A., Wang, H., Tustison, N., Avants, B., Warfield, S. K., and Landman, B. “MICCAI 2013 Seg- mentation Algorithms, Theory and Applications (SATA) Challenge Results Summary,” MICCAI 2013 challenge workshop on segmentation: Algorithms, theory and applications. (2013):

55.   Avants,  B. B.,  Tustison,  N. J.,  Song,  G.,  Cook,  P. A.,  Klein,  A.,  and Gee,  J. C. “A Reproducible  Evaluation  of ANTs Similarity Metric Performance in Brain Image Registration” Neuroimage 54, no.   3 (2011):  2033–44. doi:10.1016/j.neuroimage.2010.09.025

56. Avants, B. B., Tustison, N. J., Stauffer, M., Song, G., Wu, B., and Gee, J. C. “The Insight ToolKit Image Registration

Framework” Front Neuroinform 8, (2014): 44. doi:10.3389/fninf.2014.00044

57. Avants, B. B., Klein, A., Tustison, N. J., Woo, J., and Gee, J. C. “Evaluation of Open-Access, Automated Brain Ex- traction Methods on Multi-Site Multi-Disorder Data” 16th annual meeting for the organization of human brain mapping (2010):

58.  Das, S. R., Avants, B. B., Grossman, M., and Gee, J. C. “Registration Based Cortical Thickness Measurement”

Neuroimage 45, no. 3 (2009): 867–79. doi:10.1016/j.neuroimage.2008.12.016

59. Sluimer, I., Schilham, A., Prokop, M., and Ginneken, B. van. “Computer Analysis of Computed Tomography Scans of the Lung: A Survey” IEEE Trans Med Imaging 25, no. 4 (2006): 385–405. doi:10.1109/TMI.2005.862753

60.  Tustison, N. J., Johnson, H. J., Rohlfing, T., Klein, A., Ghosh, S. S., Ibanez, L., and Avants, B. B. “Instrumentation Bias in the Use and Evaluation of Scientific Software:  Recommendations  for Reproducible Practices in the Computational Sciences” Front Neurosci 7, (2013): 162. doi:10.3389/fnins.2013.00162

61. Rikxoort, E. M. van and Ginneken, B. van. “Automated Segmentation of Pulmonary Structures in Thoracic Com- puted Tomography Scans: A Review” Phys Med Biol 58, no. 17 (2013): R187–220. doi:10.1088/0031-9155/58/17/R187

62. Doel, T., Gavaghan, D. J., and Grau, V. “Review of Automatic Pulmonary Lobe Segmentation Methods from CT”

Comput Med Imaging Graph 40, (2015): 13–29. doi:10.1016/j.compmedimag.2014.10.008

63.  Warfield, S. K., Zou, K. H., and Wells, W. M. “Simultaneous  Truth and Performance Level Estimation (STA- PLE): An Algorithm for the Validation of Image Segmentation” IEEE Trans Med Imaging 23, no. 7 (2004): 903–21.

doi:10.1109/TMI.2004.828354

64. Coxson, H. O., Rogers, R. M., Whittall, K. P., D’yachkova, Y., Paré, P. D., Sciurba, F. C., and Hogg, J. C. “A Quantification of the Lung Surface Area in Emphysema Using Computed Tomography” Am J Respir Crit Care Med 159, no. 3 (1999):

851–6. doi:10.1164/ajrccm.159.3.9805067

65. Perez, A., 4th, Coxson, H. O., Hogg, J. C., Gibson, K., Thompson, P. F., and Rogers, R. M. “Use of CT Morphometry to

Detect Changes in Lung Weight and Gas Volume” Chest 128, no. 4 (2005): 2471–7. doi:10.1378/chest.128.4.2471

66. Coxson, H. O. and Rogers, R. M. “New Concepts in the Radiological Assessment of COPD” Semin Respir Crit Care

Med 26, no. 2 (2005): 211–20. doi:10.1055/s-2005-869540

67. Stolk, J., Putter, H., Bakker, E. M., Shaker, S. B., Parr, D. G., Piitulainen, E., Russi, E. W., Grebski, E., Dirksen, A., Stockley, R. A., Reiber, J. H. C., and Stoel, B. C. “Progression Parameters for Emphysema:  A Clinical Investigation” Respir Med 101, no. 9 (2007): 1924–30. doi:10.1016/j.rmed.2007.04.016

68. Uppaluri, R., Hoffman, E. A., Sonka, M., Hartley, P. G., Hunninghake, G. W., and McLennan, G. “Computer Recognition

of Regional Lung Disease Patterns” Am J Respir Crit Care Med 160, no. 2 (1999): 648–54. doi:10.1164/ajrccm.160.2.9804094

69.   Xu, Y., Sonka, M., McLennan, G., Guo, J., and Hoffman, E. A. “MDCT-Based 3-D Texture Classification of Emphysema and Early Smoking Related Lung Pathologies” IEEE Trans Med Imaging 25, no.  4 (2006):  464–75. doi:10.1109/TMI.2006.870889

70. Gevenois, P. A., De Vuyst, P., Sy, M., Scillia, P., Chaminade, L., Maertelaer, V. de, Zanen, J., and Yernault, J. C. “Pulmonary

Emphysema: Quantitative CT During Expiration” Radiology 199, no. 3 (1996): 825–9. doi:10.1148/radiology.199.3.8638012

71. Hoffman, E. A., Simon, B. A., and McLennan, G. “State of the Art. a Structural and Functional Assessment of the Lung via Multidetector-Row Computed Tomography:  Phenotyping Chronic Obstructive Pulmonary Disease” Proc Am Thorac Soc 3, no. 6 (2006): 519–32. doi:10.1513/pats.200603-086MS

72.  Gee, J., Sundaram, T., Hasegawa, I., Uematsu, H., and Hatabu, H. “Characterization of Regional Pulmonary Me- chanics from Serial Magnetic Resonance Imaging Data” Acad Radiol 10, no. 10 (2003): 1147–52.

73. Sundaram, T. A. and Gee, J. C. “Towards a Model of Lung Biomechanics: Pulmonary Kinematics via Registra- tion of Serial Lung Images” Med Image Anal 9, no. 6 (2005): 524–37. doi:10.1016/j.media.2005.04.002

74.    Aykac,  D.,  Hoffman,  E. A.,  McLennan,  G.,  and  Reinhardt,  J. M. “Segmentation  and  Analysis  of the  Human Airway Tree from Three-Dimensional X-Ray CT Images” IEEE Trans Med Imaging 22, no.   8 (2003):  940–50. doi:10.1109/TMI.2003.815905

75. Park, W., Hoffman, E. A., and Sonka, M. “Segmentation of Intrathoracic Airway Trees: A Fuzzy Logic Approach”

IEEE Trans Med Imaging 17, no. 4 (1998): 489–97. doi:10.1109/42.730394

76. Ederle, J. R., Heussel, C. P., Hast, J., Fischer, B., Van Beek, E. J. R., Ley, S., Thelen, M., and Kauczor, H. U. “Evaluation of Changes in Central Airway Dimensions, Lung Area and Mean Lung Density at Paired Inspiratory/expiratory High-Resolution Computed Tomography” Eur Radiol 13, no. 11 (2003): 2454–61. doi:10.1007/s00330-003-1909-5

77. Lo, P., Ginneken, B. van, Reinhardt, J. M., Yavarna, T., Jong, P. A. de, Irving, B., Fetita, C., Ortner, M., Pinho, R., Sijbers, J., Feuerstein, M., Fabijańska, A., Bauer, C., Beichel, R., Mendoza, C. S., Wiemker, R., Lee, J., Reeves, A. P., Born, S., Weinheimer, O., Rikxoort, E. M. van, Tschirren, J., Mori, K., Odry, B., Naidich, D. P., Hartmann, I., Hoffman, E. A., Prokop, M., Pedersen, J. H., and Bruijne, M. de.  “Extraction of Airways from CT (EXACT’09)” IEEE Trans Med Imaging 31, no.  11 (2012):

2093–107. doi:10.1109/TMI.2012.2209674

78.  Feuerstein, M., Kitasaka, T., and Mori, K. “Adaptive Branch Tracing and Image Sharpening for Airway Tree

Extraction in 3-d Chest CT” Proc. of second international workshop on pulmonary image analysis (2009):

79. Lee, J. and Reeves, A. P. “Segmentation of the Airway Tree from Chest CT Using Local Volume of Interest” Proc. of second international workshop on pulmonary image analysis (2009):

80.  Song, G., Tustison, N., and Gee, J. C. “Airway Tree Segmentation by Removing Paths of Leakage” Proc. 3rd int. workshop pulmonary image analysis (2010): 109–116.

81. Staring, M., Xiao, C., Shamonin, D. P., and Stoel, B. C. “Pulmonary Vessel Segmentation Using Vessel Enhance- ment Filters” VESsel SEgmentation in the Lung (2012):

82. Ostergaard, L., Weisskoff, R. M., Chesler, D. A., Gyldensted, C., and Rosen, B. R. “High Resolution Measurement of Cerebral Blood Flow Using Intravascular Tracer Bolus Passages. Part I: Mathematical Approach and Statisti- cal Analysis” Magn Reson Med 36, no. 5 (1996): 715–25.

83.  Ohno, Y., Hatabu, H., Murase, K., Higashino, T., Kawamitsu, H., Watanabe, H., Takenaka, D., Fujii, M., and Sugimura, K. “Quantitative Assessment of Regional Pulmonary Perfusion in the Entire Lung Using Three-Dimensional Ultrafast Dynamic Contrast-Enhanced Magnetic Resonance Imaging: Preliminary Experience in 40 Subjects”

J Magn Reson Imaging 20, no. 3 (2004): 353–65. doi:10.1002/jmri.20137

84. Hansch, A., Kohlmann, P., Hinneburg, U., Boettcher, J., Malich, A., Wolf, G., Laue, H., and Pfeil, A. “Quantitative Eval- uation of MR Perfusion Imaging Using Blood Pool Contrast Agent in Subjects Without Pulmonary Diseases and in Patients with Pulmonary Embolism” Eur Radiol 22, no. 8 (2012): 1748–56. doi:10.1007/s00330-012-2428-z

85. Yushkevich, P. A., Piven, J., Hazlett, H. C., Smith, R. G., Ho, S., Gee, J. C., and Gerig, G. “User-Guided 3D Active Con- tour Segmentation of Anatomical Structures: Significantly Improved Efficiency and Reliability” Neuroimage 31, no. 3 (2006): 1116–28. doi:10.1016/j.neuroimage.2006.01.015

86. Caselles, V., Kimmel, R., and Sapiro, G. “Geodesic Active Contours” Int J Comput Vision 22, (1997): 61–79.

87. Zhu, S. and Yuille, A. “Region Competition: Unifying Snakes, Region Growing, and Bayes/MDL for Multiband

Image Segmentation” IEEE Trans Pattern Anal Mach Intell 18, no. 9 (1996): 884–900.

88. Available at http://www.itksnap.org

89.  Dregely, I., Mugler, J. P., 3rd, Ruset, I. C., Altes, T. A., Mata, J. F., Miller, G. W., Ketel, J., Ketel, S., Distelbrink,  J., Hersman,  F. W., and Ruppert,  K. “Hyperpolarized  Xenon-129  Gas-Exchange  Imaging of Lung Microstructure: First Case Studies in Subjects with Obstructive Lung Disease” J Magn Reson Imaging 33, no.  5 (2011):  1052–62. doi:10.1002/jmri.22533

90.   Mugler,  J. P., 3rd, Altes, T. A., Ruset,  I. C., Dregely,  I. M., Mata, J. F., Miller,  G. W., Ketel, S., Ketel, J., Hersman, F. W., and Ruppert,  K. “Simultaneous  Magnetic  Resonance  Imaging of Ventilation  Distribution  and Gas Up- take in the Human Lung Using Hyperpolarized Xenon-129” Proc Natl Acad Sci U S A 107, no.  50 (2010): 21707–12. doi:10.1073/pnas.1011912107

91. Qing, K., Mugler, J. P., 3rd, Altes, T. A., Jiang, Y., Mata, J. F., Miller, G. W., Ruset, I. C., Hersman, F. W., and Ruppert, K. “Assessment of Lung Function in Asthma and COPD Using Hyperpolarized 129Xe Chemical Shift Saturation Recovery Spectroscopy and Dissolved-Phase MRI” NMR Biomed 27, no. 12 (2014): 1490–501. doi:10.1002/nbm.3179

92.   Altes, T. A., Mata, J., Lange, E. E. de, Brookeman,  J. R., and Mugler, J. P., 3rd.   “Assessment  of Lung Develop- ment Using Hyperpolarized  Helium-3 Diffusion MR Imaging” J Magn Reson Imaging 24, no.  6 (2006):  1277–83. doi:10.1002/jmri.20723

93. Woodhouse, N., Wild, J. M., Paley, M. N. J., Fichele, S., Said, Z., Swift, A. J., and Beek, E. J. R. van. “Combined Helium-

3/proton Magnetic Resonance Imaging Measurement of Ventilated Lung Volumes in Smokers Compared to

Never-Smokers” J Magn Reson Imaging 21, no. 4 (2005): 365–9. doi:10.1002/jmri.20290

94. Altes, T. A., Powers, P. L., Knight-Scott, J., Rakes, G., Platts-Mills, T. A., Lange, E. E. de, Alford, B. A., Mugler, J. P., 3rd, and Brookeman, J. R. “Hyperpolarized 3He MR Lung Ventilation Imaging in Asthmatics: Preliminary Findings” J Magn Reson Imaging 13, no. 3 (2001): 378–84.